# TP6 Interfaçage de moteurs pas à pas avec Arduino UNO

Moteur pas à pas

Introduction

Le moteur pas à pas est un moteur à courant continu sans balai qui divise l'angle de rotation complet de 360 ​​° en plusieurs étapes égales.

Le moteur tourne en appliquant une certaine séquence de signaux de commande. La vitesse de rotation peut être modifiée en modifiant la vitesse à laquelle les signaux de commande sont appliqués.

# Schéma d'interface

Moteur pas à pas d'interface avec Arduino UNO

# Exemple

Faire tourner le moteur pas à pas alternativement dans le sens des aiguilles d'une montre et dans le sens contraire des aiguilles d'une montre.

Ici, nous utilisons un moteur pas à pas unipolaire à six fils. Seuls quatre fils sont nécessaires pour contrôler ce moteur pas à pas. Les deux fils de prise centraux du moteur pas à pas sont connectés à une alimentation 5V.

Le pilote ULN2003 est utilisé pour entraîner le moteur pas à pas.

| void setup() { pinMode(4, OUTPUT); pinMode(5, OUTPUT); pinMode(6, OUTPUT); pinMode(7, OUTPUT); } void loop() { /* Rotation in one direction */ for(int i = 0; i<12; i++) { digitalWrite(7, HIGH); digitalWrite(6, LOW); digitalWrite(5, LOW); digitalWrite(4, LOW); delay(100); digitalWrite(7, HIGH); digitalWrite(6, HIGH); digitalWrite(5, LOW); digitalWrite(4, LOW); delay(100); digitalWrite(7, LOW); digitalWrite(6, HIGH); digitalWrite(5, LOW); digitalWrite(4, LOW); delay(100); digitalWrite(7, LOW); digitalWrite(6, HIGH); digitalWrite(5, HIGH); digitalWrite(4, LOW); delay(100); digitalWrite(7, LOW); digitalWrite(6, LOW); digitalWrite(5, HIGH); digitalWrite(4, LOW); delay(100); digitalWrite(7, LOW); digitalWrite(6, LOW); digitalWrite(5, HIGH); digitalWrite(4, HIGH); delay(100); digitalWrite(7, LOW); digitalWrite(6, LOW); digitalWrite(5, LOW); digitalWrite(4, HIGH); delay(100); digitalWrite(7, HIGH); digitalWrite(6, LOW); digitalWrite(5, LOW); digitalWrite(4, HIGH); delay(100); } digitalWrite(7, HIGH); digitalWrite(6, LOW); digitalWrite(5, LOW); digitalWrite(4, LOW); delay(100); | /* Rotation in opposite direction */ for(int j = 0; j<12; j++) { digitalWrite(7, LOW); digitalWrite(6, LOW); digitalWrite(5, LOW); digitalWrite(4, HIGH); delay(100); digitalWrite(7, LOW); digitalWrite(6, LOW); digitalWrite(5, HIGH); digitalWrite(4, HIGH); delay(100); digitalWrite(7, LOW); digitalWrite(6, LOW); digitalWrite(5, HIGH); digitalWrite(4, LOW); delay(100); digitalWrite(7, LOW); digitalWrite(6, HIGH); digitalWrite(5, HIGH); digitalWrite(4, LOW); delay(100); digitalWrite(7, LOW); digitalWrite(6, HIGH); digitalWrite(5, LOW); digitalWrite(4, LOW); delay(100); digitalWrite(7, HIGH); digitalWrite(6, HIGH); digitalWrite(5, LOW); digitalWrite(4, LOW); delay(100); digitalWrite(7, HIGH); digitalWrite(6, LOW); digitalWrite(5, LOW); digitalWrite(4, LOW); delay(100); digitalWrite(7, HIGH); digitalWrite(6, LOW); digitalWrite(5, LOW); digitalWrite(4, HIGH); delay(100); } digitalWrite(7, LOW); digitalWrite(6, LOW); digitalWrite(5, LOW); digitalWrite(4, HIGH); delay(100); } |
| --- | --- |

- #include <Stepper.h>

- Stepper(steps, pin1, pin2)

- Stepper(steps, pin1, pin2, pin3, pin4)

- setSpeed(rpm)

- step(steps)